Microsoft Word Author Guidelines for CVPR Proceedings

Polina SOLOVEVA	Tyler Marino

M2 AI, Computer Vision Course

Abstract

The ABSTRACT is to be in fully justified italicized text, at the top of the left-hand column, below the author and affiliation information. Use the word “Abstract” as the title, in 12-point Times, boldface type, centered relative to the column, initially capitalized. The abstract is to be in 10-point, single-spaced type. Leave two blank lines after the Abstract, then begin the main text. Look at previous CVPR abstracts to get a feel for style and length.

Introduction

Malaria remains a critical global health challenge, particularly in low-resource regions. According to the World Health Organization [1], early and accurate diagnosis is essential for effective treatment. The gold standard for diagnosis is the visual examination of thin blood smears under a microscope. Clinicians must identify infected Red Blood Cells (RBCs) and calculate the parasitemia rate (the percentage of infected cells relative to the total number of cells). This manual process is time-consuming, tedious, and prone to human error, especially when analyzing thousands of cells per patient.

To automate this task, researchers have recently applied Deep Learning. For instance, Kassim et al. [2] proposed a state-of-the-art architecture combining U-Net and Faster R-CNN. While highly accurate, such models require significant computational power, which is often unavailable in remote or low-resource medical facilities.

Prior to the Deep Learning era, classical computer vision methods offered computationally efficient solutions. Tek et al. [3] demonstrated that the Green channel of RGB images provides the optimal contrast for separating RBCs from the background. To handle the problem of overlapping cells, Das et al. [4] successfully applied the Watershed algorithm, while Di Ruberto et al. [5] utilized morphological operations and granulometry. Although these methods are sometimes less robust to noise than neural networks, they consume significantly less memory and processing power.

In this work, we propose a frugal automated pipeline for parasitemia estimation. We revisit classical segmentation techniques and evaluate them against the ground truth from the Kassim et al. dataset [2]. Our goal is to demonstrate that optimized classical methods can achieve a necessary balance between accuracy and efficiency, making them suitable for deployment on mobile devices and low-power laptops in developing countries.

Dataset and Preprocessing

Dataset Specification

This study utilizes the publicly available NIH-NLM-ThinBloodSmearsPf dataset described by Kassim et al. [2]. The data was collected at the Medical College Hospital in Bangladesh. It consists of Giemsa-stained thin blood smear images obtained from 193 patients, including both infected individuals and healthy controls. A smartphone camera attached to a light microscope eyepiece captured the images. The image resolution is 5312 x 2988 pixels.

Ground Truth Annotations

The dataset is divided into two subsets. The "Point Set" includes images from 160 patients with cells marked by single coordinates. The "Polygon Set" covers 33 patients where cells are outlined with polygonal boundaries. Both subsets contain labels for Uninfected and Parasitized RBC. In this study, we exclusively utilized the Polygon Set boundary annotations for the evaluation of segmentation accuracy, and labels for the classification accuracy.

Segmentation Preprocessing Pipeline

Raw microscopy images often contain noise and uneven illumination. Therefore, we applied a standardized preprocessing pipeline. First, we extracted the Green channel from the original RGB images. Previous studies [3] demonstrated that the Green channel offers the highest contrast between erythrocytes and the background. Second, we applied Contrast Limited Adaptive Histogram Equalization (CLAHE). This method corrects lighting variations across the circular region of interest and enhances local details without amplifying noise. This prepared image served as the input for our segmentation experiments.

Methodology: Segmentation

Color Space Selection

To select the best input for segmentation, we compared RGB, HSV, and Lab color spaces. The Green channel of RGB was selected because it provides the highest contrast for cell boundaries. Additionally, we chose to test the Hue channel from HSV because it represents color information independent of brightness. To clean the Hue channel, we created a binary mask from the Value channel to remove flares from the dark background. We then used a bitwise operation to isolate the Hue of the cells. Both the Green and Masked Hue channels were kept as candidates for further testing of segmentation methods.

Otsu Binarization

We started with Otsu’s Binarization as a baseline for global thresholding. This method automatically separates cells from the background by calculating an optimal intensity threshold. We performed a grid search to optimize its performance using three criteria: the smoothing level to reduce camera noise, the cleaning intensity using morphological opening to remove small artifacts, and the minimum artifact size set to filter out debris. The results showed that while Otsu’s method effectively finds cell regions, it cannot separate cells that touch each other, which leads to large connected clusters.

Circle Hough Transform

The Circle Hough Transform was used to detect cells based on their circular geometry. We tuned five key parameters through a grid search to improve accuracy. We adjusted the edge smoothness to improve boundary detection and set the cell spacing to a minimum of 25-35 pixels between centers to prevent double counting. We also balanced the detection sensitivity to ensure the algorithm catches faint cell boundaries without including background noise. Finally, the cell size limits were constrained to match the actual physical radius of erythrocytes in the microscopy images. To determine the order of parameters, we manually measured the cell diameter, approximately 140-160 pixels. This approach was the most effective method for splitting complex erythrocyte clusters.

Watershed Optimization

The Watershed algorithm was implemented to separate overlapping cells by treating the image as a topographic surface. We performed a grid search to optimize the distance mask size for smoother distance calculation and the distance threshold ratio to accurately identify the center markers of each cell. The algorithm calculates the Euclidean Distance Transform to find these markers and then floods the area to find precise boundaries where cells touch. After the initial segmentation, we applied a hole-filling morphological operation to the binary mask. This step is critical because erythrocytes often appear as ring shapes, and filling the centers ensures that no internal cell information is lost before the classification stage.

Cell Extraction for Classification

For the final classification stage, we extracted individual cells using two different strategies: the Watershed method utilized the binary masks to black out the background and crop by bounding boxes, while the Hough method used square crops with the original background to preserve the natural morphology around the circular detections. Labels were assigned by checking if ground truth points or polygon centers fell within the segmented areas; if a cluster was not fully separated, it was labeled as parasitized if at least one parasite point was detected inside it.

- References

World Health Organization, World malaria report 2022. Geneva: World Health Organization, 2022.

Y. M. Kassim et al., "Clustering-Based Dual Deep Learning Architecture for Detecting Red Blood Cells in Malaria Diagnostic Smears," IEEE Journal of Biomedical and Health Informatics, vol. 25, no. 5, pp. 1735–1746, May 2021

F. B. Tek, A. G. Dempster, and I. Kale, "Computer vision for microscopy diagnosis of malaria: a comprehensive review," Malaria Journal, vol. 8, no. 153, 2009.

D. K. Das, M. Ghosh, M. Pal, A. K. Maiti, and C. Chakraborty, "Machine learning approach for automated screening of malaria parasite using light microscopic images," Micron, vol. 45, pp. 97–106, 2013.

C. Di Ruberto, A. Dempster, S. Khan, and B. Jarra, "Segmentation of blood images using morphological operators," in Proceedings of the 15th International Conference on Pattern Recognition (ICPR), vol. 3, pp. 397–400, 2000.