Microsoft Word Author Guidelines for CVPR Proceedings

Polina SOLOVEVA	Tyler Marino

M2 AI, Computer Vision Course

Abstract

The ABSTRACT will be finished at the end.

# Introduction

Malaria remains a critical global health challenge, particularly in low-resource regions. According to the World Health Organization [1], early and accurate diagnosis is essential for effective treatment. The gold standard for diagnosis is the visual examination of thin blood smears under a microscope. Clinicians must identify infected Red Blood Cells (RBCs) and calculate the parasitemia rate (the percentage of infected cells relative to the total number of cells). This manual process is time-consuming, tedious, and prone to human error, especially when analyzing thousands of cells per patient.

To automate this task, researchers have recently applied Deep Learning. For instance, Kassim et al. [2] proposed a state-of-the-art architecture combining U-Net and Faster R-CNN. While highly accurate, such models require significant computational power, which is often unavailable in remote or low-resource medical facilities.

Prior to the Deep Learning era, classical computer vision methods offered computationally efficient solutions for segmentation. Tek et al. [3] demonstrated that the Green channel of RGB images provides the optimal contrast for separating RBCs from the background. To handle the problem of overlapping cells, Das et al. [4] successfully applied the Watershed algorithm, while Di Ruberto et al. [5] utilized morphological operations and granulometry. Although these methods are sometimes less robust to noise than neural networks, they consume significantly less memory and processing power.

Following segmentation classification approaches are necessary to determine first if a blood cell is infected with malaria thus allowing for a classification of parasitemia (ratio of blood cells infected with malaria). Training methods using classical machine learning approaches such as Logistic Regression [7] and Random Forest [8] models are lightweight and quite successful with feature engineering. More modern deep learning architectures are developed for lightweight edge tasks such as the shufflenet_v2_x1_0 model [9] which come pre-trained of the shelf with weights from ImageNet1k_v1 for example. The convolutional nature of this deep learning allows one to avoid extracting feature values and lets the model learn directly from the images input.

In this work, we propose a frugal automated pipeline for parasitemia estimation. We revisit classical segmentation techniques and evaluate them against the ground truth from the Kassim et al. dataset [2]. Our goal is to demonstrate that optimized classical methods can achieve a necessary balance between accuracy and efficiency, making them suitable for deployment on mobile devices and low-power laptops in developing countries.

# Dataset and Preprocessing

## Dataset Specification

This study utilizes the publicly available NIH-NLM-ThinBloodSmearsPf dataset described by Kassim et al. [2]. The data was collected at the Medical College Hospital in Bangladesh. It consists of Giemsa-stained thin blood smear images obtained from 193 patients, including both infected individuals and healthy controls. A smartphone camera attached to a light microscope eyepiece captured the images. The image resolution is 5312 x 2988 pixels. Pre-segmented data which can be found on Kaggle [6] of ~27,000 images with ~13,500 of each labeled as parasitic and non-parasitic cells which was distilled from the NIH dataset of Blood Cell Plates is also used for model building.

## Ground Truth Annotations

The dataset is divided into two subsets. The "Point Set" includes images from 160 patients with cells marked by single coordinates. The "Polygon Set" covers 33 patients where cells are outlined with polygonal boundaries. Both subsets contain labels for Uninfected and Parasitized RBC. In this study, we exclusively utilized the Polygon Set boundary annotations for the evaluation of segmentation accuracy, and labels for classification accuracy.

## Segmentation Preprocessing Pipeline

Raw microscopy images often contain noise and uneven illumination. Therefore, we applied a standardized preprocessing pipeline. First, we extracted the Green channel from the original RGB images. Previous studies [3] demonstrated that the Green channel offers the highest contrast between erythrocytes and the background. Second, we applied Contrast Limited Adaptive Histogram Equalization (CLAHE). This method corrects lighting variations across the circular region of interest and enhances local details without amplifying noise. This prepared image served as the input for our segmentation experiments.

# Methodology: Segmentation

## Color Space Selection

To select the best input for segmentation, we compared RGB, HSV, and Lab color spaces. The Green channel of RGB was selected because it provides the highest contrast for cell boundaries. Additionally, we chose to test the Hue channel from HSV because it represents color information independent of brightness. To clean the Hue channel, we created a binary mask from the Value channel to remove flares from the dark background. We then used a bitwise operation to isolate the Hue of the cells. Both the Green and Masked Hue channels were kept as candidates for further testing of segmentation methods.

## Otsu Binarization

We started with Otsu’s Binarization as a baseline for global thresholding. This method automatically separates cells from the background by calculating an optimal intensity threshold. We performed a grid search to optimize its performance using three criteria: the smoothing level to reduce camera noise, the cleaning intensity using morphological opening to remove small artifacts, and the minimum artifact size set to filter out debris. The results showed that while Otsu’s method effectively finds cell regions, it cannot separate cells that touch each other, which leads to large connected clusters.

## Circle Hough Transform

The Circle Hough Transform was used to detect cells based on their circular geometry. We tuned five key parameters through a grid search to improve accuracy. We adjusted the edge smoothness to improve boundary detection and set the cell spacing to a minimum of 25-35 pixels between centers to prevent double counting. We also balanced the detection sensitivity to ensure the algorithm catches faint cell boundaries without including background noise. Finally, the cell size limits were constrained to match the actual physical radius of erythrocytes in the microscopy images. To determine the order of parameters, we manually measured the cell diameter, approximately 140-160 pixels. This approach was the most effective method for splitting complex erythrocyte clusters.

## Watershed Optimization

The Watershed algorithm was implemented to separate overlapping cells by treating the image as a topographic surface. We performed a grid search to optimize the distance mask size for smoother distance calculation and the distance threshold ratio to accurately identify the center markers of each cell. The algorithm calculates the Euclidean Distance Transform to find these markers and then floods the area to find precise boundaries where cells touch. After the initial segmentation, we applied a hole-filling morphological operation to the binary mask. This step is critical because erythrocytes often appear as ring shapes, and filling the centers ensures that no internal cell information is lost before the classification stage.

## Cell Extraction for Classification

For the final classification stage, we extracted individual cells using two different strategies: the Watershed method utilized the binary masks to black out the background and crop by bounding boxes, while the Hough method used square crops with the original background to preserve the natural morphology around the circular detections. Labels were assigned by checking if ground truth points or polygon centers fell within the segmented areas; if a cluster was not fully separated, it was labeled as parasitized if at least one parasite point was detected inside it.

# Methodology: Classification

For the training of Logistic Regression, we ran a grid search test regularization strength ('clf__C') [0.01, 0.1, 1, 10, 100], number of iterations ('clf__max_iter') [500, 1000], loss function ('clf__penalty’) [‘l1’, ‘l2’, ‘elasticnet’], optimization algorithms ('clf__solver') ['lbfgs', 'liblinear', 'saga']. For the training of the Random Forest  models we ran a grid search test across number of trees (‘n_estimators') [100, 300], the maximum depth of the trees ('max_depth' ) [10, 20], the maximum features subsampled that each tree intakes ('max_features') ['sqrt', 'log2'], the minimum samples to split ('min_samples_split') [2, 5, 10], the samples for each leaf node ('min_samples_leaf') [1, 2, 4], and finally if bootstrapping is to be carried out ('bootstrap') [true, false]. These were trained on a 5-fold cross validation schema using 50% training and 50% testing which had been found to be ideal in previous tests.

Again, to test for overfitting, different training runs ranging from [10, 30, 50, 70, 90] percent of the data were performed tested on 5% of the total set of data. To input into the model, feature vectors were designed by a comparison of class specific statistics. The final feature vector that was chosen was statistics surrounding a 3-means clustering of the image. This included 16 features (clustering values, pixel counts of each cluster, variance between clusters and variance within each cluster). As most malaria infected cells had a portion of the cell that was a more vibrant purple, the k-means clustering would be able to identify this pattern and then the two models would be able to find the differentiating values within the feature vector.

Fine tuning of the pretrained shufflenet_v2_x1_0 model with weights from ImageNet1k_v1 consisted of freezing of all weights except the final fully connected layer (1024, 2) as this task was a simple binary classification task so the learned embeddings of the image was not necessary to adapt. We trained this model on 30% of the raw data and performed a grid search over epochs (2, 3, 5), learning rate (0.01, 0.001, 0.0001), and batch size (16, 32, 64) to find an optimal model.

Then taking our results out of domain from the Kaggle input data into the segmented data using ground truth segmentation and the segmentation methods we propose, we test again how the transferability of our models perform.

# Experiments and Results

## Segmentation Efficiency and Accuracy

## Classification on Ground Truth

The Kaggle dataset delivered good results for our model when we stayed within the domain of the dataset, however when we stepped beyond into other segmentation methods including our own segmentation attempts, the transferability of the models was not profound. The results of the modeling efforts primarily concern the model’s ability to perform on the Kaggle Dataset, while a small portion is dedicated to the transferability.

### Performance on Kaggle Data

Selected for comparison were the strongest models for each Linear Regression, Random Forest and Shuffle Net as well as the fastest run time Shuffle Net. In the classical machine learning approaches the optimal hyperparameter tuning for the logistic regression occurred at {'clf__C': 100, 'clf__max_iter': 500, 'clf__penalty': 'l2', 'clf__solver': 'lbfgs'} and for the random forest {'bootstrap': False, 'max_depth': 20, 'max_features': 'sqrt', 'min_samples_leaf': 2, 'min_samples_split': 5, 'n_estimators': 300}

Then we test across different training sizes as the number of learnable features is set to 16 and the number of available data is ~30,000 so there is a concern of overfitting. With the discovery of a superior performance by both the Logistic Regression and the Random Forest at a training size of 50%, we were able to reduce the training time and avoid overfitting.

We see relatively consistent performance for the Logistic Regression regardless of the training size, but there is a peak of performance at 50% training size with an F1 score of 0.91. The training time of Logistic Regression is the smallest when we compare across models and training size.

Random Forest shows significant gains from initial tests at training sizes of 10% to 30% and a very slight decline to 50% of the dataset before falling at 70% of the data. At 50% data input into the training the F1 score is stronger to that of the strongest fine-tuned Shuffle Net model 0.94. When comparing the two models, the training time of the Random Forest is far smaller than that of the Shuffle Net. Random Forest has the best recall score and outperforms with an F1 score of .94 the best Shuffle Net with a F1 score of .93.

The grid search performed for shuffle net when using 30% of the training data can be understood through Figure 3. The various dimensions: batch size, epochs and learning rate are shown with respect to training time and accuracy. As expected, the training time grows roughly linear with epochs and a possibly linear performance increase with additional epochs. The learning rate is by far the greatest indicator of quality of the model. Learning rate at scales of 1/100 are significantly better than 1/1000 and 1/10000 regardless of the number of epochs and batch size. The accuracy dependent on batch size with the bigger learning rate is consistently maximized with a size of 32, whereas the smaller learning rates perform best with a smaller batch size of 16. The batch size of 32 and 64 take roughly the same training time, but the smaller batch size of 16 takes longer.

To compare the grid searched optimum with the best Logistic Regression and Random Forest, we train one Shuffle Net model with the same hyper parameters on 50% of the training set. The training time ~5x but the performance falls short of the optimum of the grid searched model.

The overall performance of models are compared through precision and recall measuring the quality of true predictions and ability of the model to find the true positives respectively. In figure 2, we can see that the best scores for precision are achieved by random forest models, but for recall Some of the Shuffle Net grid search preform the best. The Logistic Regression does not perform well in either recall score or precision.

### Out of Domain data Performance

Initial results indicate that moving to out of domain data induces significantly more false positives. We notice a relative stability in recall but a severe reduction precision to values between .13 and .35 for both the random forest and logistic regression models tested on ~2,500 segmented cells. The Shuffle Net seems to not be able to transfer at all resulting in an accuracy score of .36. The f1 score for all models drops below .3.

(Figure 1) Comparison across models of F1 scores vs training size (a), Training time vs training size (b), (precision, recall, and F1) across the best key models (c) and a final report of scores and training time for each of the best preforming models (d).

(Figure 2)

(Figure 3) Understanding how each component of the grid search affects the performance of the model at testing time.

## System Performance on Segmented Data

## Final Parasitemia Estimation

There will be final table comparison between ideal and segmented data

# Discussion

Classification:

The lightweight models, especially the random forest perform best across the training and testing set indicating accurate feature engineering and hyperparameter tuning can save time and computational resources allowing for edge applications to exist in the field.

Classification and analysis of many blood cells can be greatly reduced by using intelligent models to differentiate the space between blood cells of the type parasitic and non-parasitic, however blood cell images may come in forms that are not equivalent at each sample. There are often differences in lighting or plasma solution makeup which shifts the domain of the input images for blood cells possibly reducing a model’s ability for correct identification. Therefore, it is critical to further develop methods that are robust to out of domain blood cell samples and therefore able to be widely used in the field.

Overall, classification efforts can be improved with online learning protocols that finetune the model as they come in. For feature engineering improvements, one could identify which types of cells give the highest false positive rates and false negative rates and try to synthesize some features that are common across the false prediction spaces.

# References

World Health Organization, World malaria report 2022. Geneva: World Health Organization, 2022.

Y. M. Kassim et al., "Clustering-Based Dual Deep Learning Architecture for Detecting Red Blood Cells in Malaria Diagnostic Smears," IEEE Journal of Biomedical and Health Informatics, vol. 25, no. 5, pp. 1735–1746, May 2021

F. B. Tek, A. G. Dempster, and I. Kale, "Computer vision for microscopy diagnosis of malaria: a comprehensive review," Malaria Journal, vol. 8, no. 153, 2009.

D. K. Das, M. Ghosh, M. Pal, A. K. Maiti, and C. Chakraborty, "Machine learning approach for automated screening of malaria parasite using light microscopic images," Micron, vol. 45, pp. 97–106, 2013.

C. Di Ruberto, A. Dempster, S. Khan, and B. Jarra, "Segmentation of blood images using morphological operators," in Proceedings of the 15th International Conference on Pattern Recognition (ICPR), vol. 3, pp. 397–400, 2000.

Arunava, I. Cell Images for Detecting Malaria. Kaggle, 2018, https://www.kaggle.com/datasets/iarunava/cell-images-for-detecting-malaria.

Aliyu, A., S. Yau, and F. Musa. “Predicting Malaria Using Logistic Regression Model.” International Letters of Journal of Computer Science and Information Technology (ILJCSIT), vol. 4, 2021, pp. 1–6.

Amarianda, Naza, Eva Darnila, and Lidya Rosnita. “Classification of the Number of Malaria Cases in Asahan Regency Using Random Forest Application.” Journal of Applied Informatics and Computing, vol. 9, no. 4, Aug. 2025, pp. 1490–1497, https://doi.org/10.30871/jaic.v9i4.9960.

Zhang, Xiangyu, Xinyu Zhou, Mengxiao Lin, and Jian Sun. ShuffleNet: An Extremely Efficient Convolutional Neural Network for Mobile Devices. 2017. arXiv, arXiv:1707.01083, https://arxiv.org/abs/1707.01083.